Scattering from one screw dislocation

This document serves to define a general physical and numerical framework for understanding and modelling the scattering from individual dislocations. As such focus is on the simplest case of a screw dislocation aligned with the diffraction vector, G.

It also serves specifically to provide a model to compare to imaging data on Diamond, where the dislocation line (and hence Burgers vector) is titled by 35 deg with respect to G.

1. Geometry

Geometry of screw dislocation parallel to G

Assume a single straight screw dislocation along z-axis in the middle of an otherwise perfect single crystal. Then the displacement, u, is only along the z-direction. Here are the equations for the displacement and the strain

The displacement is only along the z-axis:

uz= b θ2π= b2π atanyx	(1)

If we assume the crystal symmetry to be simple cubic with a lattice constant d, then the positions of the atom defined by indices (k,l,m) wrt to Origo become

rklm= kd,  ld,  md+d2πatanl/k  	(2)

When viewed along the z-direction the 2D- lattice in (x,y) is perfect. Furthermore, it is axially symmetric around z.

The strain components are all 0 except

ezx= d uzd x= b2π 11+yx2-yx2= -b2π yx2+y2= -b2π sinθr ;

ezy= d uzd y= b2π 11+yx21x= b2π xx2+y2= b2π cosθr	(3)

(εxz= ezx+exz2 evidently gets a 4 in the denominator, but it is e we measure. )

Hence, locally wrt point (x0, y0) the displacement becomes

uzx,y-uzx0,y0= ezxx-x0 + ezyy-y0= b2πr -x-x0sinθ+y-y0cosθ

Fig 2.  Plot of eps = ezx(with axis rotated by 180 deg).

Geometry of screw dislocation at an angle to G

Let the sample system (x, y, z) be defined by G. Assume the line of the dislocation is in the (x, z) plane of the sample system and define a local system (x’, y, z’) with z’ along the dislocation line.  Let the angle between G and B be . We introduce the coordinate transform

xz=cosβsinβ-sinβcosβx'z'

Furthermore, without loss of generality, we can assume z = 0. With these definitions

uz=uz'cosβ = b2πatanyx'cosβ= b2πatanyxcos(β)cosβ  (4a)

ux=uz'sinβ = b2πatanyx'sinβ = b2πatanyxcos(β)sinβ  (4b)

ezx= d uzd x= -b2π y cos⁡(β)(xcosβ)2+y2	(4c)

ezy= d uzd y= b2π x cos2⁡(β)(xcosβ)2+y2	(4d)

exx= d uxd x= -b2π y sin(β)(xcosβ)2+y2	(4e)

exy= d uxd y= b2π xcosβsin(β)(xcosβ)2+y2	(4f)

All components not mentioned are 0.

There is most likely an elegant way to express this using vectors ez (unit vector along z) and B.

Dark field microscopy set-up

One issue is the coherence length. The transverse coherence length is very good, while the longitudinal is given by the energy band lcoh=ΔEEλ, which is approximately 1 mu.

Another issue is the extinction depth (1/e in intensity), which is defined in [1] as

Λ=14mdvcr0F		(5)

For Diamond (111) we have d-spacing d= 2.059 Å, m=1, volume of unit cell: vc = 178.2 Å3 , F = 8 f(sin()/)  = 8*f(0.35) ~8*1.9  and r0 = 2.8 10-5 Å. Hence, 10 mu.

We can distinguish between three cases:

- Thin foils.  Whether in Bragg or Laue, there will be no issue with dynamical scattering. The region of the sample seen by one pixel on detector may be coherent.

- Section topography on a single crystal. In this case the incoming beam is a line beam. There will be issues with dynamical scattering. The region of the sample seen by one pixel on detector may be coherent.

- A wide open beam on a single crystal in topo-tomo mode. The intensity arriving at a pixel on the detector comes from a narrow rod though the sample. Hence, transverse to the rod direction we can assume coherence, but along the rod things add up incoherently.  There will be issues with dynamical scattering.

In the following we will primarily be concerned with case 3. We note that the description in terms of a “rod“ superficially looks like the column approximation central to TEM [2, chapter 25 p 405]. However in the TEM case the incoming beam is of size Å and there is no depth resolution: so here the column is in direction incident beam. In our case, we use a wide beam but can characterize in 3D: hence our column/rod is in the direction of the diffracted beam.

In the following we shall typically assume the incoming beam is along the y-direction, and that scanning qy corresponds to a rocking scan.

Experimental details

For the numerical work we will compare to an experiment conducted at beamline ID06. A Si(111) Bragg-Bragg monochromator defined a 17 keV x-ray beam (= 0.728 Å).

The sample was a high quality artificially grown diamond plate with a thickness of 300 m. It was mounted in a transmission Laue geometry, using an in-plane 111 reflection for the study. Diamond is fcc with an edge length of 3.567 Å. This implies d=3.5673=2.059 Å , G=2πdπ= 3.051 Å-1 and  = 10.19 deg. Screw dislocations have Burgers vectors b =121 1 0= 3.5672=2.522 Å. Hence, they are placed at an angle of 0.6155 rad = 35.2 deg from the diffraction vector. The corresponding Pendullösung length, and Darwin width are g = 35 m and wgθ  = 0.0119 mrad (FWHM), respectively.

The incoming beam was condensed by a transfocator (Vaughan et al. 2011) at a distance of 38.7 m from the source. This was configured with seven one-dimensional Be lenses with an apex radius of curvature of 200 m and an effective aperture of 860 m (FWHM?). With the instrument placed 58 m from the source the divergence of the incoming beam is 0.04 mrad. Hence, the incoming divergence dominates the over the Darwin width. Our best estimate is therefore a  Gaussian resolution function qy = 0.00012 Å-1 (FWHM).

A slit 4 m from the sample defined a relatively homogenous field of 0.3 mm x 0.3 mm on the sample. Hence, the full rod is illuminated.

The beam was diffracted in the horizontal plane. The objective comprised N = 72 identical parabolic Be lenslets, each with a radius of curvature of 50 m and thickness of T= 2 mm. With distances of d1 = 0.382 m and d2 = 5.420 m , the magnification becomes M =16.2, the numerical aperture NA (rms)≈ δμ Nf MM+1 = 1.18E-646 7220 1617= 0.286  mrad; NA (FWHM)= 0.672 mrad, and the radius-of-interest roi = 327 µm. The direct space resolution, as defined by the diffraction limit is FWHMnm. The far-field detector comprises a scintillator coupled by microscope optics to a FRELON 2k x 2k CCD with a physical pixelsize of 15 µm. The objective and eyepiece in the camera implied an inherent magnification by 10, which in combination with the magnification in the x-ray objective resulted in an effective pixelsize of 93 nm. The detector was binned 2x2. From these it seems that a box-shaped spatial resolution function Rdir = 186 nm may be used as a simple approximation  for the spatial resolution.

The NA defines the resolution in the two other dimensions of reciprocal space: Δqrad  =GNA2tan(θ) = 0.0058 Å-1 (FWHM) and  Δqazim,= GNAsin2θ = 0.0060 Å-1 (FWHM).

The rocking scan performed was in steps of 0.003 deg = 0.052 mrad.

2. Formalism based on incoherent and kinematical scattering

Formalism

As mentioned above we shall start by assuming the dislocation is parallel to the G-vector and along the z-axis. If we consider the lattice locally in the vicinity of point (x0, y0) as homogeneous then we can calculate the reciprocal lattice corresponding to For simplicity let us look along a line of constant x. In direct space:

a

y

c

r



x



Fig 3. Sketch that shows that tanφ=ca=b2πrcos⁡(θ), cf also Eq 4.

In the (y,z) plane the lattice is distorted as follows

d’’

d



Fig 4. definition of d’. We have y horizontal and z upwards.

The length of the direct space vector becomes shorter: d’ =d cos(). And it is tilted with components in y and z as  (d’sin(), d’cos()). This direct space vector corresponds to a reciprocal space vector in the same direction (direct and reciprocal space rotates together) with a length of 2πd'.

Let reciprocal space be defined by  G + q, where G is the Bragg peak related to the undisturbed lattice (no dislocation). Then

qy,qz=2πd'sinϕ, 2πd'cosϕ-2πd= Gtanφ, 0= Gb2πrcosθ, 0

Due to the cylinder symmetry also in reciprocal space, in general we have

qx, qy,qz= -Gb2πrsinθ,Gb2πrcosθ, 0    (7)

We recognize the G B contrast mechanism. In practice we have a resolution of 100 nm or worse, implying that qG≤10-3 From this also follows that within experimental error G+q= G.

Main properties

In the context of the microscopy set up this means that for the screw dislocation along z:

- The scattering is characterized by a precesion of the G-vector in the (qx, qy) plane. For y=0, the G-vector is offset in direction qy, for x=0, G-vector is offset in direction qx.

- There is no contrast in varying The objective will integrate fully over the signal in direction qz.

- For fixed x, the maximum qy appears at x=y, and is Gb4πx. For x≥ 100 nm, this is an order of magnitude smaller than the resolution qx.

- If properly aligned in the lab coordinate system the q-vector is constant during a topo-tomo rotation.

Integrated intensity

First we note that along the rod at distance x from the core we can invert Eq 3 to get (with C being short for b/2):

y=±Cxqy-x2.    (8a)

This is valid for qy∈-Cx,  Cx.  Differentiating this with respect to qy gives

dy= ∓Cx2qy2Cxqy-x2 dqy. (8b)

The total length of the line segment falling L(x, qy) within the interval dqy is therefore given by L= 2dy (the sum of a y>0 and a y<0 segment).

Within the incoherent scattering picture, we can determine the integrated intensity I(x, qy) by simply integrating the area in the (x,y)-plane that is within the instrumental resolution function.

Let us first assume that the instrumental resolution is ideal.

Fig 5. Intensity as fct of x with qy as parameter. (Erroneously here b= 1.45E-10 is used.)

The curve above shows sharp lines in the real space image at a distance of x ~ b/(2qy) from the core They are not self-similar; at high qy the peaks are not so sharp. However, for x>100 nm, they are very sharp.

Fig 6. Intensity as fct of qy with x as parameter. (Erroneously here b= 1.45E-10 is used.)

The reverse plot of intensity as fct of qy for a set of positions, x, is shown above. Sharp peaks are seen at qy ~ b/(2x).

Let us assume the spatial resolution is given by a normalized function f, and the qy resolution by another normalised function g. Then

Ix,qy= C-∞∞f(qy-u)du-∞∞g(x-x')Lx',udx' 
=2C-∞∞g(x-x')dx'-∞∞f(qy-u)x'u2bx'2πu-x'2du

This simple formalism has the problem that the L→∞ when qy→0, and hence the area also goes to Infinity.  Furthermore the negative square root must be avoided. A more careful job with the limits leads to (with T being the full length of the rod) – for x’ > 0:

Ix,qy=2C-∞∞g(x-x')dx'ulowuhighf(qy-u)x'u2bx'2πu-x'2du     (9)

ulow=bx'2π(x'2+(T/2)2) ; uhigh=b2πx'

For x’ < 0 one must swap uhigh and ulow.   For x in the range where one probes both positive and negative x’ s the interval needs to be broken up in parts.

Notably the effect of these terms are different:

- introducing a final spatial  resolution Rdir evidently broadens all peaks in Fig 5 with the same amount. But in Fig 6 the high qy peaks is much more broadened.

- Introducing a final width in qy evidently broadens all peaks in Fig 6 with the same amount, but broadens the peaks in Fig 5 progressively more with increasing x.

Generalization G not parallel to B

We probe the change in G. Wrt Fig 4 the angle  becomes smaller, while the contraction of the direct space lattice along x is irrelevant.  Hence, we are left with the two components ezx and ezy as before. Repeating the exercise above we now invert Eq 4d in stead of Eq 3. Inserting the result in the equivalent of Eq 8b we obtain

dy= ∓Cx2qy2Cxqy-x2cos2 (β)dqy.

Hence, except from a new factor cos2() in front, the expression for the scattered intensity is the same as for the untilted case!  Eq 9 is still valid.

Comparison to experiment

x

G, z

x’

B, z’

Fig 7. Shown is one image in the experimental rocking scan experimental data. The coordinate system is illustrated. The AOI is around 14 µm.

Fig 7 shows a zoom on actual data. 1D line profiles were made in direction x. Extrapolating from 80 pixel to 500 points, the step in these 1D scans is 28.8 nm.

Fig 8. Overview of experimental data. The x-scans inserted represent the AOI, that is they have a range of 14 m.

Inspection of Fig 8 reveals that

- There is a noticeable contrast (a peak) out to 5 images away from the center in the rocking scan, that is out to 0.26 mrad or qy = 7.9 10-4 Å-1. Inserting in x ~ b/(2) we get 160 nm. Her mangler måske et cos beta eller lignende

- The widths of the peaks are of order 1 µm, substantially larger than the calculated spatial resolution. Inspection of the lower right corner of the figure to the left in Fig 7 suggests that the actual resolution is substantially better than this.

Eq 9 has been implemented in MATLAB, using the function integral2. We use the following settings:  fu=12π Δqyexp-0.5u∆qy2 with qy =0.00012 Å-1,  g(x) = Box-fct with a FWHM of Rdir = 180 nm., T = 300 µm, b = 2.522 Å.

Read in of profiles: matlab file: read_exp_data.m

function [intensity] = int_intensity(x0,eps0)

C = 2.522/(2*pi);

sigx = 1E-7;  %in m

sig_eps = 0.5E-4;

Npoint = 200;

if (abs(eps0) - 4*sig_eps < 0)

intensity = 0;

return

end

dx = 4*sigx/Npoint;

d_eps = 4*sig_eps/Npoint;

sum_int = 0;

for ii=1:2*Npoint

x = x0 + dx*(ii-Npoint);

f =  exp( -(x-x0)^2/(2*sigx^2) );

for jj=1:2*Npoint

eps = eps0 + d_eps*(Npoint-jj);

g = exp( -(eps-eps0)^2/(2*sig_eps^2) );

dummy1 = C*x/eps - x^2;

if dummy1>0

sum_int = sum_int + f*g*abs(x)/eps^2/sqrt(dummy1)*dx*d_eps;

end

end

end

intensity = C/(4*pi*sigx*sig_eps)*sum_int;

end

3. Formalism based on coherent and kinematical scattering

The analysis performed above will not be valid close to the core of the dislocation as the reciprocal space is not homogeneous within the coherence volume in that case. As the scattering from close to the core is of prime interest to us, we need to do better.

We shall use the classical procedure to determine the scattering power S for a small crystal [1]. For ease of presentation we shall assume a simple cubic lattice with lattice constant d comprising only one species of atoms. The incoming beam will be along a constant x trajectory. One issue in setting up the formalism is that the incoming wavefront is easiest to describe in the lab coordinate system, the reciprocal space in a the sample system -  with the dislocation axis along z - while the rod is easiest to define in detector space.

We are going to use repeatedly the fact that

-U/2U/2expi u qu du 2= sin2Uqu/2sin2qu/2~ UsinUqu/2Uqu/22=U2sinc 2(Uqu/2).	(10)

The HWHM of the sinc squared is 1.392, see this link, hence FWHM(qu) = 2*2*1.39/U= 5.56/U.

In the sample system, at position Q = G + q in reciprocal space we have (with G being a Bragg lattice point,  rklm the unperturbed direct space lattice and C a constant):

SG+q=C f2k=K1K2l=L1L2m=M1(l)M2(l)exp(-i 2πmd sinθλ) expi (G +q).rklm0+uklm2  (11)

Here the first term takes care of the phase shift due to the incoming beam being tilted at angle . Applying Braggs law:

= C f2k=K1K2l=L1L2m=M1(l)M2(l)exp(-i  mπ) expi (G +q).rklm0+uklm2

Note that indices m depends on l – as the rod is at an angle with the sample system. Now we can substitute the atoms shifted in in the rod that is centered around z = tan()y with atoms centered around z =0 if we add a phase shift of exp(-i 2πmd sinθλ)= exp(-i mπ). Combined with the term we found above we have multiply of 2. As a result

SG+q= C f2k=K1K2l=L1L2m=M1M2 expi G +q.rklm0+uklm2   (12)

As we have many atoms within the inspected volume we can without loss of precision go to a continuous description. Entering the rod around (x0, y0, 0) we have

SG+q, x0, y0= C f2d6-Tx2+x0Tx2+x0expi xqxdx –Ty2+y0Ty2+y0expi yqy+iG+qzd2πatanyx dy-Tz2Tz2expi zqzdz2

= C f2d6-Tx2+x0Tx2+x0expi xqxdx –Ty2+y0Ty2+y0expi yqy+iG+qzd2πatanyx dy2Tz2sinc2qzTz2           (13)

Hence, qz is centered on 0 with a FWHM of 5.56/Tz. For Tz≥100 nm, it appears we can simplify to

SG+q, x0, y0= C f2d6

-Tx2+x0Tx2+x0expi xqxdx –Ty2+y0Ty2+y0expi yqy+iGd2πatanyx dy2Tz2sinc2qzTz2      (14)

It is instructive to consider the homogenous case above once more around (x0, 0). This implies that atanyx ~atanyx0~yx0. Hence,

SG+q, x0, 0= C f2d6Tx2Ty2Tz2sinc2qxTx2sinc2Ty2qy+Gd2πx0 sinc2qzTz2      (15)

In short this is a product of three delta-functions, centered at (qx, qy, qz) = (0,-Gd2πx0,0).

Intensity in dark field microscopy experiment

The instrument has a finite resolution in reciprocal space. If we assume this to be an anisotropic Gaussian with rms widths (qx , qy, qz), the intensity I becomes

Iq, x,y = C -∞∞-∞∞-∞∞Sqx-u,qy-v, qz-w,x,yexp-12uΔqx2-12vΔqy2-12wΔqz2du dv dw 	(16)

Notably, we are taking data in the “rotation mode” which mathematically implies a change in coordinate system and a Jacobian matrix in going from S to I. In other words this introduces a Lorentz factor. We shall neglect this factor as it only is a scale factor when we are looking at the vicinity of one Bragg peak at one energy.

Above we learned that the NA of the objective is sufficiently large to integrate over the qx and qz..  In a handwaving way I propose things add up coherently when integrating over qz and qy but incoherently when integrating over qx.

Combining Eqs. 14 and 16 we have the following

Ipartqy,x,y=C'-∞∞du-∞∞exp-12qy-vΔqy2dv-Rdir2+xRdir2+xexpi x'udx' –lcoh2+ylcoh2+yexpi y'v+iGd2πatany'x' dy'2	(17)

Finally we need to integrate along the entire rod, that is the illuminated diffracted beam path projected unto the (x,y) plane in the sample system, T/cos, with T being the “thickness” in the lab system.



cos(





This gives the equation we have been looking for:

N= round12Tlcohcosθ-1 ; I(qy,x)=C'n=-NN-∞∞du-∞∞exp-12qy-vΔqy2dv-Rdir2+xRdir2+xexpi x'udx' n-12lcohn+12lcohexpi y'v+iGd2πatany'x' dy'2       (18)

In future the four parameters in this equation should be measured with good precision: the spatial resolution Rdir, the coherence length lcoh , the resolution of the rocking curve, qy, and the illuminated thickness, T.

We remark that both the spatial resolution and the coherence length are treated as box functions. To be more precise this should be replaced by Gaussians or similar. Likewise the beam profile may not be a box function.

Alternative optics

An increased NA will improve the spatial resolution, and allow the strain field closer to the core to be studied. There are no adverse effects: the objective will still integrate over the entire distribution in both qx and qz.

Numerical simulation of actual data

The aim is to calculate the intensity for say 15  x  positions in the range [0.1 1.5] m and for say  20 qy positions covering the range []. For each of these we wish to sum 2N +1 = 1E-41E-6cosθ  ~102 points along the rod.  In total this implies performing 15*20*100 = 30,000 4D integrals.

MATLAB has the built in function integral3, but not a specific function for 4D integrals.

have generated the function screw_scat(qx, qy, qz) based on Eq 5 in MATLAB.

Reference

- Jens Als Nielsen and Des MacMarrow. Elements of modern x-ray physics.

- D. B. Williams, B.C. Carter. Transmission X-ray microscopy. Plenum 1996.